Smart Vehicles Control - Project Work

| Team |  | Date |  |
| --- | --- | --- | --- |
| Members |  | Additional files |  |

DAY 1 PROJECT UNIT

Longitudinal control

Project outcome: Implement and tune the PI cruise controller in student_controller.py; use repeatable CSV evidence to justify the final parameters.

1. Short trivia: key terms

| No. | Term | Explain it in one sentence or give the key equation |
| --- | --- | --- |
| 1 | Reference speed / setpoint |  |
| 2 | Control error |  |
| 3 | Feedback |  |
| 4 | Proportional gain Kp |  |
| 5 | Integral action and anti-windup |  |

2. Closed-loop control diagram

Draw the complete loop. Label: v_ref, summing point, speed error e_v, PI controller, throttle/brake, vehicle, measured speed v, feedback arrow, and road-slope disturbance.

Drawing / inserted figure:

Prediction: which road section will create the largest speed error, and why: ____________________________________________________________________
____________________________________________________________________________

DAY 1 - RESULTS PAGE

3. Work with the Python simulator

Editable file: courses/day_1/python_3d_adas/student_controller.py. Keep the supplied steering controller and simulator model unchanged.

cd courses\day_1\python_3d_adas
python run_simulator.py --headless --controller student --duration 60 --csv results\day1_baseline.csv

Planned Kp range: ____________________________________________________________________

Planned Ki range: ____________________________________________________________________

Chosen anti-windup rule: ____________________________________________________________________

4. Controlled tuning

Method: set Ki = 0 and tune Kp first; freeze Kp; then add a small Ki; change one decision per run.

| Run | Kp | Ki | Rise time (s) | Overshoot (%) | Final error (m/s) | Uphill max error (m/s) | Decision / reason |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |
| Final |  |  |  |  |  |  |  |

python run_simulator.py --headless --controller student --duration 90 --csv results\day1_final.csv

5. Final scorecard and conclusion

| Criterion | Pass condition | Result | Pass / fail |
| --- | --- | --- | --- |
| Rise time | 90% target in <= 10 s |  |  |
| Overshoot | < 10% |  |  |
| Final speed error | < 0.25 m/s |  |  |
| Uphill error | <= 1.0 m/s after recovery |  |  |
| Command logic | No simultaneous throttle and brake |  |  |
| Road keeping | No road departure |  |  |

Engineering conclusion: what changed, what improved measurably, and what trade-off remains: ____________________________________________________________________
____________________________________________________________________________
____________________________________________________________________________

Submitted files: controller, baseline CSV, final CSV, screenshot/plot: ____________________________________________________________________

DAY 2 PROJECT UNIT

Lateral control

Project outcome: Complete a full lap at 12 m/s by tuning speed-dependent Pure Pursuit look-ahead while balancing tracking accuracy, steering activity, and lateral acceleration.

1. Short trivia: key terms

| No. | Term | Explain it in one sentence or give the key equation |
| --- | --- | --- |
| 1 | Kinematic bicycle model |  |
| 2 | Cross-track error ey |  |
| 3 | Look-ahead distance Ld |  |
| 4 | Heading-to-target angle alpha |  |
| 5 | Lateral acceleration |  |

2. Pure Pursuit drawing

Draw the vehicle and target geometry. Label: rear axle, vehicle heading, look-ahead target, Ld, alpha, wheelbase L, steering direction, and the circular arc to the target.

Drawing / inserted figure:

Write the Pure Pursuit steering equation and explain the sign of alpha: ____________________________________________________________________
____________________________________________________________________________

Prediction: what will happen if Ld is too short? What if it is too long: ____________________________________________________________________
____________________________________________________________________________

DAY 2 - RESULTS PAGE

3. Work with the Python simulator

Editable file: courses/day_2/python_3d_adas_day2/student_controller.py. Keep the validated Day 1 gains unchanged.

| Candidate | Ld0 (m) | Kv (s) | Ld at 6 m/s | Ld at 12 m/s | Ld at 16 m/s |
| --- | --- | --- | --- | --- | --- |
| Fixed baseline | 6.0 | 0.0 |  |  |  |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| Final |  |  |  |  |  |

4. Tuning results

| Run | Max ¦ey¦ (m) | Mean ¦ey¦ (m) | Steering-rate RMS (deg/s) | Peak ay (m/s2) | Lap progress (%) | Road departure | Decision |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |
| Final |  |  |  |  |  |  |  |

python run_simulator.py --headless --controller student --duration 75 --target-speed 12 --csv results\final_full_lap.csv

5. Selection and sensitivity check

Selected Ld0 and Kv, with the two metrics that justify the choice: ____________________________________________________________________
____________________________________________________________________________

| Speed | Look-ahead | Max ¦ey¦ | Steering-rate RMS | Peak ay | Interpretation |
| --- | --- | --- | --- | --- | --- |
| 8 m/s |  |  |  |  |  |
| 16 m/s |  |  |  |  |  |

Engineering conclusion: main look-ahead trade-off and one simulator limitation: ____________________________________________________________________
____________________________________________________________________________

DAY 3 PROJECT UNIT

Integrated ADAS, testing, and robustness

Project outcome: Integrate ACC with the Day 1 and Day 2 controllers, then freeze the controller and evaluate it against repeatable sensor, actuator, and lateral disturbances.

Repository mapping: Use the current Day 3 package for ACC tuning, then the current Day 4 package for robustness testing. Both activities belong to Day 3 of the revised three-day course.

1. Short trivia: key terms

| No. | Term | Explain it in one sentence or give the key equation |
| --- | --- | --- |
| 1 | Desired gap ddes = d0 + Th v |  |
| 2 | Closing speed |  |
| 3 | Time to collision (TTC) |  |
| 4 | Target arbitration |  |
| 5 | Supervisor and SAFE_STOP |  |
| 6 | Noise, bias, delay, dropout, authority loss |  |

2. Integrated controller architecture

Draw the data and command flow. Include: driver/road/ACC targets, minimum selector, safety-state or health supervisor, Day 1 PI to throttle/brake, Day 2 Pure Pursuit to steering, vehicle, lead-vehicle sensor, and feedback signals.

Drawing / inserted figure:

Explain why the lowest active speed constraint wins: ____________________________________________________________________
____________________________________________________________________________

DAY 3 - RESULTS PAGE

3. Part A - Tune integrated ACC

Editable file: courses/day_3/python_3d_adas_day3/student_controller.py. Keep Day 1 PI and Day 2 Pure Pursuit parameters fixed.

python run_simulator.py --headless --controller student --duration 105 --target-speed 14 --csv results\acc_candidate.csv

| Run | d0 (m) | Th (s) | Kd (1/s) | Kclosing | Min gap (m) | Min TTC (s) | Speed RMSE | Decision |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline | 6.0 | 1.5 | 0.20 | 0.50 |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |
| Final |  |  |  |  |  |  |  |  |

Selected ACC settings and safety-efficiency trade-off: ____________________________________________________________________

4. Part B - Freeze and test robustness

Evaluation package: courses/day_4/python_3d_adas_day4. Do not modify faults.py, evaluate_project.py, scenario timing, or thresholds.

python evaluate_project.py --controller student --duration 105 --target-speed 14 --csv results\day3_robustness_final.csv

| Scenario | Prediction: main effect | Min true gap | Max lane error | Progress | Safe stop? | Pass / fail | Evidence / explanation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Nominal |  |  |  |  |  |  |  |
| Radar dropout |  |  |  |  |  |  |  |
| Lateral push |  |  |  |  |  |  |  |
| Brake fade |  |  |  |  |  |  |  |
| Combined |  |  |  |  |  |  |  |

Acceptance gates: 0 collisions; 0% road departure; minimum true gap > 3.0 m; maximum lane error <= 1.60 m; progress >= 95%; SAFE_STOP appears in radar-fault cases.

Final conclusion: scenarios passed, worst-case scenario and metric, safety benefit, performance cost, and validity limitation: ____________________________________________________________________
____________________________________________________________________________
____________________________________________________________________________